**********************************************************

*	BD Software C Compiler	v1.31			 *

*	On_Line Hanbook					 *

*	written by 	Leor Zolman,			 *

*			BD Software			 *

*			9/15/79, 12/17/79,		 *

*			2/1/80, 3/8/80			 *

*							 *

**********************************************************

The lunatic is in my head

The lunatic is in my head

You raise the blade, you make the change

You re-arrange me 'till I'm sane

You lock the door

And throw away the key

There's someone in my head but it's not me.

-- PINK FLOYD

++++++++++++++++++++++++++++++++++++++++++++++++++++++++++

+	Note that a DYNAMIC OVERLAY scheme has been	 +

+	newly included in version 1.31...		 +

+	See NEWSTUFF.DOC (either on disk, by the name    +

+	NEWSTUFF.DOC, or on paper in the form of the	 +

+	1.3 and 1.31 doc updates), OVERLAYS.DOC and	 +

+	the CLINK section of this document for more	 +

+	information.					 +

+	THE v1.3 USER'S GUIDE DOES NOT CONTAIN ALL OF	 +

+	THE INFORMATION GIVEN IN THIS FILE AND IN THE	 +

+	OTHER .DOC FILES (AND/OR PAPER UPDATE SHEETS.)	 +

+	MAKE SURE TO READ ALL OF THESE .DOC FILES TO	 +

+	PICK UP INFO ON THE NEW	SWAPPING SCHEME AND	 +

+	OTHER FEATURES!!!!!				 +

++++++++++++++++++++++++++++++++++++++++++++++++++++++++++

Welcome to the world of Structured High-Level

Microcomputer Programming!! This document is intended as

a quick guide to using the BDS C Compiler package.

=================================================================

=	Note to TRS-80 owners and other users having an		=

=	upper-case only console I/O device: Two programs	=

=	have recently been added to the package, CONVERT.C	=

=	and CC0T.C, to allow you to represent those characters	=

=	not normally available on your keyboards. See the	=

=	last part of this file for details.			=

=================================================================

The main components of the BDS C package are: four

executable COM files, a standard library file and a skeletal

run-time subroutine module.

A description of each follows:

CC1:	Because C loads the entire source file into memory

in one shot, the compilation is broken up into two

phases (not "passes", strictly; The two phases end

up taking about 8 "passes" to actually implement),

maximizing the amount of memory available for the

source file.

CC1, the first phase of the compiler, accepts a C

source file with any filename and extension (say,

"foo.c") and produces a temporary file containing

a symbol table and an encoded form of the source code.

By default, version 1.3 will leave the temporary

file in RAM and attempt to load in the next phase of

the compiler (CC2) from the currently logged in disk.

The "v" option, if given on the CC1 command line, will

instead force the temporary file to be written to disk

will the extension ".CCI". This file is then read in

by the second phase of the compiler when you

invoke CC2.

The file extension ".C" is NOT assumed for the

input file, so saying "foo" when you mean "foo.c"

would not work.

If the source file name is preceded by a disk

designation, then the input is taken from the specified

disk, and the output (if any) is written to that disk.

If any errors are detected during CC1, then neither

will CC2 be auto-loaded nor will any .CCI file

be written to disk.

Following the name of the source file on the command

line, you may specify a list of options preceded by

a single dash ("-") with no spaces between the

options.

The recognized options are:

s 	causes undeclared identifiers to be implicitly

declared as int variables, wherever possible.

(this is not especially useful, since most

C compilers demand that all variables be

declared always. If you desire portability,

don't use this option.)

p	causes the result of the preprocessor phase

of compilation (i.e., the result of #define

and #include text substitutions) to be

written to the CP/M console output device,

with lines numbers generated.

hex digit (5-f)  sets symbol table size to the

specified value (in K bytes);

default is 8 (6 for versions x.xT.)

v		Forces .CCI file to be written to disk

(instead of auto-loading CC2.COM)

w, x, y, or z	Causes CC2.COM to be auto-loaded from

disk A, B, C or D respectively after

CC1 finishes processing (note

that the letters a, b, c, and d are

already spoken for by the symbol table

size option above.)

You'd use one of these when the disk

on which your BDS C commands reside

is NOT the currently logged disk;

In the absence of any of these

options (and assuming "v" wasn't given)

CC1 will auto-load CC2 from the

currently logged disk.

Examples:

A>cc1 foobar.c -s6

supresses errors for undefined variables, sets symbol

table size to 6K bytes, and auto-loads CC2 from the

currently logged disk;

A>cc1 zot.c -evp

sets symbol table size to 14K bytes (e hex) and

forces ZOT.CCI to be written to the currently logged

disk. The text is displayed on the console after

all #defines and #includes have been processed.

A>cc1 b:bunny.c -y

gets BUNNY.C from disk B, and auto-loads CC2.COM

from disk C (if there are no bugs in BUNNY.) If

the y option were omitted, then the file BUNNY.CCI

would get written to disk B (the same disk the source

came from.)

On an 8080, processing speed (not counting disk

waiting) is about 12 lines of source/second.

On a 4 MHz Z80, crunch speed is about 30 lines/sec.

CC2:	This is the second phase of the compiler. CC2 accepts

a ".CCI" file as input, and writes out a ".CRL" file

if no errors are detected. (CRL is mnemonic for

'C ReLocatable'.) Normally CC2.COM is automatically

loaded in (by CC1) and operates on the CCI file left

in memory by CC1; If invoked explicitly, then the

.CCI file is loaded from disk.

If all goes well, writing out of the CRL file is

followed by deletion of the "CCI" file (if it

exits), and compilation is complete.

Like CC1, if a disk is specified explicitly as in

A>cc2 c:yahoo

then the .CCI file is loaded from the specified disk

and the .CRL file is written to that same disk.

On an 8080, execution speed = about 35 lines/sec.

On a 4 MHz Z80, speed is about 90 lines/sec.

CLINK:	This program links a "main" function from some

CRL file together with C.CCC (for common system

subroutines) and any subordinate functions which

"main" may require (from perhaps many CRL files).

Alternately, CLINK may be used to prepare a code

segment which can be swapped in at run time by the

root module.

A successful linkage causes a ".COM" file to be

generated. At this point, the 8080 absolute

machine code file is ready to be executed (for

better or worse) as a transient command by CP/M

(or swapped in by another COM file.)

The first argument on the command line must be the

name of a CRL file containing a "main" function. If

the name is specified with an extension, then that

extension is interpreted specially as indicating

which disks are to be involved in the operation (this

is akin to the mechanism ASM uses to determine source

and destination disks.)

For example, if the first argument to CLINK were

given as:

A>clink foo.bc

then CLINK would interpret the "b" in ".bc" as

specifying the disk on which "DEFF.CRL" and "C.CCC" are

to be found, and the "c" in ".bc" as specifying which

disk the .COM file is to be written to. Both of these

values, if omitted, default to the currently logged in

disk.

The first argument may also be preceded by a disk

designation, to state where all .CRL files (except

DEFF.CRL) are to be searched for (by default.)

For example, the command:

A>clink b:zot.ac

tells CLINK to get C.CCC and DEFF.CRL from disk A; to

write the ouput file to disk C; and to find ZOT.CRL on

disk B.

If a third character is given in the file extension,

and it is an "s", then the file specified is treated as

a segment to be prepared for swapping in, and before CLINK

does anything else, it will ask you to enter the load

address (the address at which the segment will be loaded into

memory by use of the "swapin" function) from the console. Once

this is done, and followed by a carriage return, then CLINK

will proceed with the linkage without yanking in the run-time

module (C.CCC.) See way below for some segment examples.

Note that if this option is used, then the first two code

letters of the file extension must also be given, even

if they are only to assume their default values. This is

necessary because it's pretty tough to specify the third

letter of the extension without also specifying the first

two!

Any other CRL files to search may also be specified

on the command line (WITHOUT their ".CRL" extensions),

causing these to be searched in the order specified.

The default disk to search will be the same disk from

which the original CRL file was taken; this default

can be overridden by specifying an explicit disk

designation for any appropriate CRL file name needing

it. For example,

A>clink c:foo.bb bar a:zot fraz

causes disk C to be searched for the files FOO.CRL,

BAR.CRL and FRAZ.CRL, while disk A would be searched

to find ZOT.CRL. Disk B is where CLINK would expect

DEFF.CRL and C.CCC to reside, and the output would go

to disk B also.

After all given CRL files have been searched once,

CLINK will automatically search DEFF.CRL.

If there are still some unresolved references, then

CLINK will ask for input from the keyboard to try

resolving them. This situation is likely to arise

in the case where a function becomes "needed"

AFTER the CRL file containing it has already

been searched. When yanking functions out of a CRL

file, CLINK will only take those which are already

known to be needed (due to a prior reference.)

Thus, it will save you some pain to order the CRL

filenames on the command line in such a way as to

minimize undefined backward references. This ONLY

applies to multiple CRL files; the order of functions

within a single CRL file makes no difference.

There are also several options which may be

specified on the CLINK command line. Each option must

be preceded by a dash (-); the space between

options and their argument (if needed) is optional.

(Yes, this is different than the CC1 option format.)

The presently supported options are:

-s		Prints out load statistics;

-d		(New for version 1.31) Use this to run

the code immediately after linking,

for debugging purposes, instead of

writing a COM file out to disk.

Note that no arguments can be passed

to the program when executed in this way.

-t nnnn		Reserves location nnnn (hex) and

above for user; default is to

reserve no space. What this really

does is to cause the first op in

the object file to be

lxi sp,nnnn

instead of

lhld RAM+6

sphl

-o name		Causes the .COM file generated to

have the given name. Default is

the name of the first .CRL file

given (the one with the "main"

function.)

-e xxxx		Sets start of data area to address

xxxx, to maintain consistency between

several separate .COM files when

chaining (via the library function

"exec") is used and external data

need to be shared.

If the program will be swapping in

segments, then the value given MUST BE

GREATER THAN THE HIGHEST ADDRESS THAT

WILL EVER BE OCCUPIED BY SWAPPED-IN

CODE.

-c		OBSOLETE. DO NOT USE.

-y name		(New for version 1.31)

Yanks in the symbol table file "name.SYM"

from disk, and defines all the symbols

found therein. This is used when linking

an overlay segment, to pre-define the function

addresses resident in the root node and

accessible by the segment. See the -w

option below.

-w		(New for version 1.31)

Causes a SID-compatible symbol table

file to be written to disk for use

either by SID or by CLINK later on

when linking overlay segments (see the -y

option above.)

Examples:

A>clink foo bar

gets "main" from the file FOO.CRL, searches for

needed functions first in FOO.CRL and then, if needed,

in BAR.CRL and DEFF.CRL. All files are assumed to

reside on the currently logged in disk.

A>clink b:ihtfp belle -s

searches for IHTFP.CRL and BELLE.CRL on disk B; prints

a statistics summary when linkage is complete. The

files DEFF.CRL and C.CCC are assumed to reside on the

currently logged in disk; output also goes to the

currently logged in disk.

A>clink b:ihtfp.aa -s belle -o zot

is the same as the last example except: the output

file is called ZOT.COM, DEFF.CRL and C.CCC are assumed

to reside on A, and output goes to A.

A>clink stoned -t7000 -s

sets top of memory to 7000h and prints out load

statistics. Current disk used for everything.

Note that if the load statistics tell you that

the "LAST ADDRESS" is greater than the "TOP OF

MEMORY", the program hasn't got the chance of a

snowball in hell of running correctly.

A>clink rootseg -w -e 6000 -s

links "rootseg" with the intention of having "rootseg"

call in segments form disk as it runs. The symbol table

is written out to disk (via the -w option) named

"ROOTSEG.SYM", and the external data area is set to

6000 hex, indicating that the code for "rootseg"

itself ends somewhere low enough so that even with the

longest segment(s) in RAM, the last code address doesn't

exceed 6000 hex. A stat summary is also output to

the console.

A>clink seg1.aas -y rootseg -s

This assumes seg1 is going to be swapped in by the file

linked in the last example above. First CLINK will ask

the user for a "Segment address" to be entered from the

console. After it is entered, a COM file will be produces

origined at the given segment address, and the 2K run-time

package will NOT be stuck in (as it always is for non-

segments.) The symbol file "ROOTSEG.SYM" is yanked in

and milked for any usefuladdresses it may contain, and

a stat summary is printed afterwards, as usual. Note that

some of the stats are not printed (such as the external

data starting address), since they have no meaning to

a segment.

CLIB:	This program maintains .CRL files, allows transfer

of functions from one CRL file to another, etc. To

invoke CLIB, just type

A>clib

CLIB will print a line such as

Function buffer size = nnnnn

specifying the largest function size that can be

handled. Attempting to "transfer" or "extract" a

function larger than this size could be destructive.

Next CLIB will prompt with a "*". Typing "h" at

this point will give you a command summary.

Basically, you work CLIB by opening one to six

CRL files (which then become associated with

"file numbers"), diddling the files to your hearts

content, closing all files which you altered, and

typing control-C.

The old version of any CRL file you change with CLIB

is renamed to name.BRL (for Backup ReLative).

A sample session of CLIB to, say, transfer the

functions named "FOO", "BAR", and "ZOT" from

a .CRL file named "DSSR" to one named "RTS" would

go as follows:

A>clib

BD Sofware C Librarian v1.31

Function buffer size = xxxxx bytes

* open 0 dssr

* open 1 rts

* t 0 1 foo

* t 0 1 bar

* t 0 1 zot

* c 1

* ^C

A> ...

The "open" commands prepare to do work on a .CRL file,

and associate each .CRL file opened with a digit (0-5).

The "transfer" commands tell CLIB to transfer the

named function from the first file (named by file #)

to the second file (also named by number).

The "close" command need only be given for files

which have been altered; since DSSR wasn't written

to in the above example, it didn't need to be closed,

but RTS did need to be closed.

Note: using the "d" command (to delete a function from a CRL

file) does NOT free up the CRL directory space which was

taken up by the deleted function's name. This space does not

become available UNITL AFTER THE FILE IS CLOSED (using the "c"

command.) This means that if, for example, you want to delete some

stuff from DEFF.CRL in order to make room for other stuff, you

have to close DEFF.CRL after performing the deletiions, re-open

it, and THEN transfer the new functions in...otherwise you'll

draw a "No more space in CRL directory" error.

DEFF.CRL:  This file contains the standard function library...

all 70+ functions worth. See the BDS C User's Guide

and STDLIB2.C for documentation on these functions.

DEFF2.CRL: More library functions. This file is an overflow

from DEFF.CRL. Note that while DEFF.CRL is searched

automatically by CLINK, DEFF2.CRL is NOT, so "deff2"

must be stated explicitly on the CLINK command line

if any functions in DEFF2.CRL are needed.

C.CCC:	The run-time skeleton file, containing code for

processing the command line (generating argc and

argv, for you UNIX lovers), room for file I/O

buffers, some math subroutines, etc.

OTHER THINGS YOU GET WITH THE PACKAGE:

Sample source files:

****************************************************

* Many of the sample source files, having been     *

* thrown together in a hurry, may not do the things*

* you'd like them to do. SO CHANGE THEM. Anything  *

* written in C is just begging to be improved, re- *

* written, hacked up, etc. You've got the compiler;*

* if you want your bufferred I/O routines to buffer*

* up 1K before dumping instead of 128 bytes, CHANGE*

* THE SOURCE. Don't trust some fool like me to do  *

* everything right the first time. MAKE IT BETTER. *

****************************************************

STDLIB1.C	Source for library functions which

are written in C and present, in

compiled form, within DEFF.CRL.

STDLIB2.C	More library function sources.

---------------------------------------------------------

STONE.C		Game program using good recursion

techniques. Tough to beat, too. Written

by ace hackers Clark Baker and Terry Hayes.

PRESSUP.C	Another neat game -- excellent human

engineering on this one by Steve Ward.

OTHELLO.C	Game again. Doesn't put up much of a

fight, but has sentimental value as

the first real program ever compiled by

BDS C, and also serves as a good example

of passing arrays as arguments to functions.

LIFE.C		Yet another game program; shows off speed

of programs compiled by BDS C.

MM.C		Mastermind. Another goodie By Steve.

TTT.C		You guessed it -- now try to beat it!

--------------------------------------------------------

TABIFY.C	Turns spaces into tabs, optimizing

storage requirements for a text file.

COPYALL.C	A utility program, for single-disk

systems, which copies a whole mess

of files from one disk to another

while buffering stuff up in memory as

much as possible. This is the program

I wrote to make BDS C package disks, after

the number of files in the package got

large enough to make using FILECOPY.COM

a bit impractical.

PNUM.C		Utility to print out a text file with

line numbers (see also the "-p" option

to CC1.).

-------------------------------------------------------

TELNET.C	Program to let a system be used

as a terminal, optionally routing

stuff from modem to disk and vice-

versa. Nice new hanshaking and check-

summing features. Hairy.

-------------------------------------------------------

CONVERT.C	Converts unprintable characters (for

systems having upper-case only and/or

incomplete special symbol set) into

special 2-character sequences beginning

with the character '#'. See also CC0T.C

CC0T.C		Converts C source files containing the

special sequences generated by the

CONVERT program back into a form that

can be compiled.

For more info on CONVERT and CC0T, see

the last section of this file.

-------------------------------------------------------

Other stuff:

C.DOC		You seem to have found this one.

NEWSTUFF.DOC	New documentation for version 1.3 and 1.31

(May be on paper, in the form of the 1.3

and 1.31 documentation update sheets.)

OVERLAYS.DOC	Information on the overlay scheme.

-------------------------------------------------------

FLOAT.DOC,	Bob Mathias's contributed floating

FLOAT.C		point package (uses the "fp" function

which resides in DEFF2.CRL.)

FLOATSUM.C	Sample program using the floating

point package.

-------------------------------------------------------

CCRL.DOC,	Earl T. Cohen's COM-to-CRL conversion

CCRL.C		program.

BIGFIO.C	Large-buffered File I/O library, by

Scott Layson.

QUEUE.C		FIFO queue package, by Scott Layson.

-------------------------------------------------------

===== == === === = =========

Notes on the BDS C compiler:

===== == === === = =========

THIS IS NOT AN INTERPRETER.

Some hacks, such as BASIC-E, are billed as compilers

but actually just do some preprocessing and then

interpret the program. BDS C is a true compiler,

generating not-too-optimal but nevertheless quick

8080 code.

For the gory details on the BDS C implementation, see

my notes to APPENDIX A of the EXCELLENT book

"The C Programming Language."

Variable types supported:

int,

char,

unsigned,

struct,

union,

pointers to  a) all of the above

b) to other pointers and

c) to functions,

arrays (of one or two dimensions),

simple combinations of the above.

For example,

char *foo[10][15];

declares foo to be a two dimensional array of

pointers to characters;

char (*foo)();

declares foo to be a pointer to a function returning

a character;

char *foo, bar, zot[10];

declares foo to be a pointer to characters, bar to

be a single char variable, ant zot to be an array

of 10 characters.

Since all functions in C may be used recursively, all

variable accessing is done relative to a base-of-

stack-frame pointer, kept in the BC register pair

during execution. Note that it takes 8 bytes of code

to transfer a simple local variable whose address is

(Base of stack pointer) + foo

to the HL register pair; The code appears as:

lxi h,foo

dad b

mov a,m

inx h

mov h,m

mov l,a

To get an array element or structure element is even

more hairy. Facts like this are enough to make me

REALLY wish Intel had bothered to implement a double

byte indirect load instruction. Oh well.

If your keyboard doesn't support the '{' and '}'

characters (open and close brace, for those of

you whose printer doesn't know about ascii 7B and

7D), the symbols 'begin' and 'end' may be

substituted. Don't unless you have to; '{' and '}'

take up less memory.

The CONVERT program will perform this conversion,

if necessary.

Test Run

==== ===

To see if everything is OK, try compiling the

sample program STONE.C. The sequence should look

like this:

A>cc1 stone.c

BD Software C Compiler v1.31  (Part I)

xxK elbowroom

BD Software C Compiler v1.31 (Part II)

xxK to spare

A>clink stone

BD Software C Linker v1.31

Linkage complete

xxK left over

A>stone

Difficulty (1-1000):

<and so on as STONE.COM executes...>

Good luck. You're about to embark on an exciting

voyage into the land of straightforward structured

programming: "C". Good luck,

-leor

P.S. If any of you authors out there want to do some-

thing really useful, WRITE A GOOD TUTORIAL ON C.

Since the language has been used primarily in

a university-research environment up until now,

all the available texts assume for starters that

the reader lives, breathes and thinks COMPUTERS

all the time and should have no problem figuring

out this "simple, low level" language. And yet,

very intelligent people can be driven CRAZY by

stuff like: equivalence of pointers and arrays,

storage allocation, etc.

If YOU write a good FUNDAMENTAL (not BASIC) intro-

ductory book, I'LL get to work on a better

compiler...

---

Now for all you UPPER-CASE-ONLY (and/or limited symbols) people:

The C compiler itself doesn't know the difference between a

printable and a non-printable character (as far as your upper-

case-only terminal is concerned.) In order for a source file to

compile properly, characters such as "left bracket" must appear

as their ASCII value indicates. Since your terminal can't print

these special characters (there are about nine of them used),

typing out a normal source file (say, LIFE.C) will produce some

strange symbols on your terminal in place of what's really there.

What's needed is some way to encode those special symbols so that

you can enter them and read them on your terminal for editing, and

them un-encode them for purposes of compilation.

Thus, the programs CONVERT.C and CC0T.C have been provided to

translate back and forth between an encoded form of source code

in which all of these unprintable characters get represented in

terms of printable symbols; i.e., the character "#" (which has

no special meaning to the C compiler unless it is part of one of the

strings "#define", "#include", "#ifdef", "#ifndef", "#else", "#undef"

and "#endif") followed by one of the characters:

B, L, R, C, V, U, or H.

(Note that the "#undef" preprocessor directive has been just

recently added to the compiler, and clashes with the "#U"

encoding. If you need to use the "#undef" directive, you'll

need to change the "U" to some other character (perhaps "N")

by modifying the CONVERT.C and CC0T.C programs.)

The precise usage of each of the sequences #B, #L, #R, etc.,

is covered by the comments within the CONVERT.C source file.

So, the CONVERT program takes source files in the natural form

and converts them to "#" notation (let's call it ".CT" format.)

The form of the CONVERT command is:

A>convert foo.c bar.ct

where foo.c is an existing normal source file, and bar.ct is

a new file to be created as an encoded form of foo.c.

You may then edit the .ct file normally, using the "#"

conventions to represent all those special characters.

When you're ready to compile the file, the CC0T program must be

run first to convert the .CT file back into the natural format.

To reverse the example above, the command would be:

A>cc0t bar.ct zot.c

leaving zot.c as the file ready for compilation by BDS C.

If you don't have an upper-case-only terminal (or one lacking

a complete symbolic character set), then you won't need

to bother with CONVERT.C or CC0T.C. Otherwise...I suggest you

upgrade as soon as possible to a full ASCII terminal, so you

can forget about CONVERT and CC0T forever.

Incidentally, both CONVERT.C and CC0T.C must be compiled

before they can be used. If this seems confusing, remember

that the problem here is NOT an incompatibility between the

source files and the compiler, but rather one between the

source files and your terminal's ability to display them.

CONVERT.C and CC0T.C will compile in their present form no

matter what; but modifying them on a limited terminal would

require passing them through CONVERT.COM first.

<< The End >>

############################################################�O#�#�4#�#�9#�#�*#��#�##��#O=!9##��#�##�#ү#!F#�##�q#!2#��#�##��#�q##C�%#�##��##Z�%#�

#��##b�##�

#Ӂ#!*#z�#�##��#=�~#�#�a#�#��O